01.26.2014   •   Version: 1.1   •   Prepared by: Lightwellinc

Upgrading a USFS 8.0 Scan Gun to version 9.1

Prepared for:

FS & BLM Users

(Lightwell Team will help you if you have questions)

Table of Contents

You will need:	3

Upgrading a USFS 8.0 Scan Gun to version 9.1	3

# You will need:

You will need the following 3 files

- (Lightwell Team has copies of these files if you face any difficulties):

  - YantraHostList.xml

  - YantraMobileApp_WCE4_5.ARMV4.CAB

  - OnRestore.txt

You will need a Scan Gun Power/USB adapter.

You will need a Windows 7 (64 bit) machine.

# Upgrading a USFS 8.0 Scan Gun to version 9.1

Plug the Power end into an outlet.

USB into your computer. (USB 2.0 port, NOT a USB 3.0 port)

Connect the scan gun.

You should see the software automatically start to install:

  - This is standard Windows software for Mobile devices. Please ALLOW Windows to install this software. If you get any prompts say “yes”/”accept”/”I Agree”/etc…. Please DO NOT block this software from being installed in any way.

Click: Start – Search “Windows Mobile Device Center”. Open it.

You should see that it has a status of “Connected”.

IF you should see that it has a status of “Not Connected”.

  - You may have to “accept” the government warning before the scan gun will connect. (on the scan gun itself)

  - Also, it seems to only work with your USB 2.0 port on your computer. (NOT your USB 3.0 port) If unfamiliar with this concept, ask your system administrator for more information.

Click: “Connect without setting up your device”.

Click: “File Management” - “Browse the contents of your device”.

Browse to: “Application”.

  - Delete “YMAProperties.ppc.xml”.

  - Delete “YantraHostList.xml”.

  - Add “YantraMobileApp_WCE4_5.ARMV4.CAB”.

    - (Drag & Drop the file)

  - Add “YantraHostList.xml”.

    - (Drag & Drop the file)

Browse to: “StartUpCtl” – “OnRestore”:

  - Delete “YantraMobileAppWCE4.ARMV4.CAB”.

  - Add “YantraMobileApp_WCE4_5.ARMV4.CAB”.

    - (Drag & Drop the file)

  - Add “OnRestore.txt”.

    - (Drag & Drop the file)

Perform a “Cold Boot”.

Connect to a network.

  - IF you are using your USFS or BLM wireless network then login like normal.

  - Otherwise ask your system administrator for another network and have him/her connect your scan gun to this other network.

Launch the Sterling Mobile Console.

  - You will see 3 new options for 9.1 environment that you can choose from in the dropdown. Select the one you want and login with your id.

    - QA

    - Training

    - Production